# APPROVAL NOTE: Equipment Integrity Inspection Report - Ref No IR-2026-08-992A

| Source Document: | Equipment Integrity Inspection Report |
| --- | --- |
| Generated Date: | 2026-09-04 02:59:32 UTC |
| Approval Status: | DRAFT — PENDING OPERATOR APPROVAL |
| Governance: | AEGIS Autonomous Industrial Agent Core |

## Executive Summary

Quarterly inspection identifies critical issues with PUMP-104B requiring immediate maintenance and TANK-301A needing repair and monitoring. PIPE-EX-12 shows minor issues within operational tolerances. Recommendations include prioritizing pump maintenance, addressing tank corrosion, and maintaining routine monitoring protocols.

## 1. Extracted Facts and Key Findings

The following factual determinations were extracted directly from the inspection report:

PUMP-104B vibration levels exceed ISO 10816-3 threshold (7.8 mm/s RMS)

TANK-301A shell wall thickness reduced to 3.1 mm (below 4.0 mm nominal minimum)

Localized pitting corrosion detected on TANK-301A lower shell ring segment 3

PIPE-EX-12 flange gasket shows minor weeping at Joint #4

## 2. Supporting Observations

Technical context and condition assessments observed during review:

Inboard bearing temperature of PUMP-104B measured at 68C (nominal ceiling 55C)

Ultrasonic thickness survey confirms PIPE-EX-12 remaining wall thickness of 5.8 mm (minimum required 4.2 mm)

Foundation settlement of TANK-301A within allowable 5 mm limit

External protective coating on PIPE-EX-12 remains intact with no atmospheric corrosion

## 3. Recommended Actions

Proposed maintenance actions requiring human operator review and authorization:

Immediate maintenance intervention required for PUMP-104B vibration and bearing temperature

Repair and thickness monitoring required for TANK-301A shell corrosion

Routine monitoring recommended for PIPE-EX-12 flange gasket condition

Implement vibration diagnostics and thermal monitoring for PUMP-104B

## 4. Operator Sign-Off & Approval

| Reviewing Authority: | Operations & Integrity Assurance |
| --- | --- |
| Current Governance State: | DRAFT — PENDING OPERATOR APPROVAL |
| Operator Signature / Timestamp: | Pending HITL Controller Confirmation |